Application Note

RS232 SERIAL INTERFACE

COMMUNICATION PROTOCOL

# INTRODUCTION

## Purpose

The purpose of this document is to explain in detail the commands and steps that can be used to control a display via RS232C.

## Definitions, Abbreviations and Acronyms

PBS	Professional Business Solutions

RC		Remote Control

ACK		Acknowledge

NACK		Not Acknowledge

NAV		Not Available

ID		Identification

0xXX		Hexadecimal notation

# COMMAND PACKET FORMAT

## Physical Specifications

- Baud Rate : 9600

- Data bits: 8

- Parity : None

- Stop Bit : 1

- Flow Control : None

- The Pin Assignments for DB9 Female connector:

Female D-Sub 9-Pin (outside view)

| Pin # | Signal | Remark |
| --- | --- | --- |
| 1 | NC |  |
| 2 | RXD | Input to LCD Monitor |
| 3 | TXD | Output from LCD Monitor |
| 4 | NC |  |
| 5 | GND |  |
| 6 | NC |  |
| 7 | NC |  |
| 8 | NC |  |
| 9 | NC |  |
| frame | GND |  |

Note: A provided crossover cable (null modem) is needed for connection to the host controller:

Digital Signage displays use RXD, TXD and GND pins for RS-232C control. For RS-232C cable, the reverse type cable should be used.

## Communication Procedure

Control commands can be sent from a host controller via the RS232 connection. A new command should not be sent until the previous command is acknowledged. However, if a response is not received within 500 milliseconds a retry may be triggered. Every valid command receives an ACK. A command that is valid but not supported in the current implementation will be responded to with a NAV (Not Available). If the command buffer is corrupt (transmission errors) the command will be responded to with a NACK. The display operates according to the received command. If the command is a valid “Get” command, the display responds with the requested info. If the command is a valid “Set” command allowed, the display performs the requested operation.

Figure1 and Figure2 explain the mechanism of the Get and Set commands.

Note: For LAN control, the port number is 5000.

Host controller

Monitor

Get command

NACK

NAV

Get

-

report command

Command code DATA[0] = x

Command code DATA[0] = x

Command data DATA[1..N]

Get command

cycle with

report from

Monitor

Internal

processing:

collect report

data

Get requested

parameter

from Monitor

Note:

No ACK, NACK

or NAV sent

to Monitor after

reception of

Get report command

-

Note:

No ACK sent

to Host controller

after reception of

Get command

Figure 1: Explanation of mechanism of Get Command.

Host controller

Monitor

Set command

ACK

NACK

NAV

Command code DATA[0] = y

Command data DATA[1..N]

Set command

cycle with

Acknowledge

from Monitor

Acknowledge

reception of

Set command

ACK= Ok

or

NACK= Not Ok

or

NAV= Not avail

Set parameter

in Monitor

Figure 2: Explanation of mechanism of Set Command.

## Command Format

The RS232 packet format:

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | … | Data[N] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In detail:

| Number of Field | Name of Field | Description |
| --- | --- | --- |
| Byte 1 | Header | Header = 0xA6 |
| Byte 2 | Monitor ID | Monitor ID Range : 1 ~ 255, 0 = broadcast. |
| Byte 3 | Category | Category = 0x00 (fixed) |
| Byte 4 | Code0 (Page) | Reserve |
| Byte 5 | Code1 (Function) | Reserve |
| Byte 6 | Length | Length of message plus checksum code. Calculate the length from Control byte to Checksum byte. |
| Byte 7 | Data Control | Data Control = 0x01 (fixed) |
| Byte 8 | Data[0] | Command code. |
| Byte 9~Byte9+(N-1) | Data[1]~Data[N] | Data. This field can be also empty. |
| Last Byte | Checksum | Checksum. Range = 0 to 255 (0xFF). Algorithm: The EXCLUSIVE-OR (XOR) of all bytes in the message except the checksum itself. Checksum = [Header] XOR [Monitor ID] XOR … DATA[0] … XOR DATA[N] |

# MESSAGES - SYSTEM

## Communication Control

This defines the feedback command from monitor to host controller when it receives the display command from the host controller, depending on the commands availability, the command reported back to host controller can be one of the ACK(0x00), NACK(0x03) or NAV(0x04).

Note: there is no reply message when the wrong ID address is being used.

### Message-Report

| Number of Field | Name of Field | Description |
| --- | --- | --- |
| Byte 1 | Header | Header = 0x21 |
| Byte 2 | Monitor ID | Monitor ID Range : 1 ~ 255 |
| Byte3 | Category | 0x00 |
| Byte4 | Page | 0x00 |
| Byte5 | MsgLen | Length of message plus checksum code. Calculate the length from Control byte to Checksum byte. |
| Byte6 | Control | 0x01 |
| Byte7 | Data[0] | Copy the received Command code. |
| Byte8~Byte8+(N-1) | Data[1]~Data[N] | Returned data associated with command code. |
| Byte 8+N | Checksum | XOR of all byte in reply/report packet(except checksum itself). |

Example  ACK reply: (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x00 | 0x00 | 0x25 | Command is well executed. |

Example  NACK  reply: (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x00 | 0x03 | 0x26 | No this command code-Data(0), the system will reply “NACK”. |

Example  NAV reply: (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x00 | 0x04 | 0x21 | 1.Checksum error, the system will reply “NAV”. 2.No this parameter-Data(1), the system will reply “NAV”. |

# MESSAGES - GENERAL

## Platform and Version Labels

This command provides the model name of platform and the display Software version to the host controller.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA2 = Platform and Version Labels - Get |  | Request the label version. |
| DATA[1] | Label |  | 0x00 = Get the FW version 0x01 = Get model name of the platform. |

Example: Get version (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA2 | 0x00 | 0x00 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA2 = Platform and Version Label – Report |  | Request the internal label version. |
| DATA[1] to DATA[N] | Character[0] to Character[N-1] |  | 36 (0x24) characters maximum. No. of characters, N = 1 to 36 (0x24). The actual size determines the value of the message size byte. |

## Power state

This command is used to set/get the power state as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x19 = Power state - Get |  | Command requests the display to report its current power state |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x19 | 0xBC |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x19 = Power State - Report |  | Command reports Power state |
| DATA[1] | Power State |  | 0x01 = Power Off 0x02 = On |

Example: Power State On (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x19 | 0x02 | 0x3E |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x18 = Power state - Set |  | Command to change the Power state of the display |
| DATA[1] | Power state |  | 0x01 = Power Off 0x02 = On |

Example: Power State Deep Sleep (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x18 | 0x01 | 0xBB |

## User Input Control

The following commands are used to lock/unlock the Remote Control and the Local Keyboard functionality corresponding.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x1D = User Input Control – Get |  | Get the lock/unlock state |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x1D | 0xB8 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x1D = User Input Control – Report |  | Report from display of lock/unlock state |
| DATA[1] | Bit meaning: - 0 = locked - 1 = unlocked | Bit 7..2 | Not used |
|  |  | Bit 1 | Local Keyboard |
|  |  | Bit 0 | Remote Control |

Example: Lock Keyboard and unlocked Remote Control (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1D | 0x01 | 0x39 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x1C = User Input Control – Set |  | Set the lock/unlock state |
| DATA[1] | Bit meaning: - 0 = locked - 1 = unlocked | Bit 7..2 | Not used. |
|  |  | Bit 1 | Local Keyboard |
|  |  | Bit 0 | Remote Control |

Example: Unlock local Keyboard and unlock remote control (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x1C | 0x03 | 0xBD |

## Power state at Cold Start

Command is used to set the cold start power state, the cold start power state are updated and stored by this command.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA3 = Power at Cold Start - Set |  | Set Power state at Cold Start |
| DATA[1] | Power state at Cold Start |  | 0x00 = Power Off 0x01 = Forced On 0x02 = Last Status |

The value is stored and it is applied only when the display starts up from cold start power state the next time:

Power Off:

The monitor will be automatically switched to Power Off mode (even if the last status was on) whenever the mains power is turned on or resumed after the power interruption.

Forced On:

The monitor will be automatically switched to ON mode whenever the mains power is turned on or resumed after the power interruption.

Last Status:

The monitor will be automatically switched to the last status (either Power Off or On) whenever the mains power is turned on or resumed after the power interruption.

Example: Set Power state at cold start to last status (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA3 | 0x02 | 0x03 |

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA4 = Power at Cold Start - Get |  | Get Power state at Cold Start state |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xA4 | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xA4 = Power at Cold Start – Report |  | Report from Power state at Cold Start state |
| DATA[1] | - Power state at Cold Start |  | 0x00 = Power Off 0x01 = Forced On 0x02 = Last Status |

Example: Current Power state at Cold Start state: Last Status  (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xA4 | 0x02 | 0xnn |

# MESSAGES - INPUT SOURCES

## Input Source

This command is used to change the current input source.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAC = Input Source – Set |  | Command requests the display to set the current input source |
| DATA[1] | Input Source Type |  | 0x01 = VIDEO 0x01 = S-VIDEO 0x03 = COMPONENT 0x03 = CVI 2 (not applicable) 0x05 = VGA 0x05 = HDMI2 0x07 = Card DVI-D 0x07 = Display Port 0x08 = Card OPS 0x08 = USB 0x09 = HDMI 0x09 = DVI-D 0x0A = Network |
| DATA[2] | Input Source Number |  | 0x00 = VIDEO 0x01 = S-VIDEO 0x00 = COMPONENT 0x01 = CVI 2 (not applicable) 0x00 = VGA 0x01 = HDMI2 0x00 = Card DVI-D 0x01 = Display Port 0x00 = Card OPS 0x01 = USB 0x00 = HDMI 0x01 = DVI-D 0x00 = Network |
| DATA[3] | OSD Style Reserved | Bit7 | Not used. |
|  |  | Bit6 | Do not switch. Source is made current. set is updated with the details of this source; however, source change is performed. 1 = Do not switch. 0 = Switch |
|  |  | Bit2.0 | Source info. Display Style 0 = Reserved 1 = Source label |
| DATA[4] | Delay time for source change |  | Unit : 100 ms |

Example: Wait 1000ms (10*100ms), Set on DVI-D  (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x07 | 0x01 | 0xAC | 0x09 | 0x01 | 0x00 | 0x0A | 0x0F |

## Current Source

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAD = Current Source – Get |  | Command requests the display to report the current input source in use. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xAD | 0x08 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAD = Current Source – Report |  | Command reports to the host controller the current input source in use by the display. |
| DATA[1] | Input Source Type |  | 0x00 = Reserved for smartcard 0x01 = Reserved for smartcard 0x02 = Reserved for smartcard 0x03 = Reserved for smartcard 0xFD = Input Source (normal state) 0xFE = Reserved for smartcard |
| DATA[2] | Input Source Number |  | For Input Source Type: 0x00, 0x01, 0x02, 0x03 0x01…0x63 = Channel Number (only for smartcard) For Input Source Type: 0xFD 0x01 = VIDEO 0x02 = S-VIDEO 0x06 = COMPONENT 0x08 = VGA 0x09 = HSMI2 0x0A = HDMI 0x0B = DVI-D 0x0D = Display Port 0x0E = Card OPS 0x0F = USB 0x10 = Network 0xFF = Unknown |

Example: Current Input Source: HDMI (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0xAD | 0xFD | 0x0A | 0x7E |

## Auto Signal Detecting

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAF = Auto Signal Detecting – Get |  | Command requests the display to report its current Auto Signal Detecting status |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xAF | 0xBC |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAF = Auto Signal Detecting – Report |  | Command reports Auto Signal Detecting Setting |
| DATA[1] | On / Off |  | 0x00 = Off 0x01 = On |

Example: Current Display settings: Off and On (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0xAF | 0x01 | 0x8B |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xAE = Auto Signal Detecting – Set |  | Command to change the Auto Signal Detecting setting of the display |
| DATA[1] | On / Off |  | 0x00 = Off 0x01 = On |

Example: Set the Display to the fallowing: Auto Signal Detecting On  (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xAE | 0x01 | 0x0D |

# MESSAGES - VIDEO

## Video Parameters

The following commands are used to get/set video parameters as it is defined below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x33 = Video Parameters – Get |  | Command requests the display to report its current video parameters. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x33 | 0x96 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x33 = Video Parameters – Report |  | Command reports to the host controller the current video parameters of the display. |
| DATA[1] | Brightness. |  | 0 to 100 of the user selectable range of the display. |
| DATA[2] | Color. |  | 0 to 100 of the user selectable range of the display. |
| DATA[3] | Contrast. |  | 0 to 100 of the user selectable range of the display. |
| DATA[4] | Sharpness. |  | 0 to 20(0x0 ~ 0x14) of the user selectable range of the display. |
| DATA[5] | Tint (Hue) |  | -50 to 50(0x0 ~ 0x64) of the user selectable range of the display. |

Example: All video parameters are set to 0x64 (100 %)  (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x08 | 0x01 | 0x33 | 0x64 | 0x64 | 0x64 | 0x14 | 0x64 | 0x0E |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x32 = Video Parameters – Set |  | Command to change the current video parameters |
| DATA[1] | Brightness. |  | 0 to 100 of the user selectable range of the display. |
| DATA[2] | Colour. |  | 0 to 100 of the user selectable range of the display. |
| DATA[3] | Contrast. |  | 0 to 100 of the user selectable range of the display. |
| DATA[4] | Sharpness. |  | 0 to 20(0x0 ~ 0x14) of the user selectable range of the display. |
| DATA[5] | Tint (Hue) |  | -50 to 50(0x0 ~ 0x64) of the user selectable range of the display. |

Example: Set all video parameters to 0x37 (55 %) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Data[5] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x08 | 0x01 | 0x32 | 0x37 | 0x37 | 0x37 | 0x37 | 0x37 | 0xAB |

## Picture Format

This command is used to control the display screen format.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x3B = Picture Format – Get |  | Command requests the display to report its current picture format |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x3B | 0x9E |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x3B = Picture Format – Report |  | Command report to the host controller the current picture format of the display. |
| DATA[1] | Picture Format | Bit 7..4 | Not used. |
|  |  | Bit 3..0 | Picture Format. 0x00 = 4:3 0x01 = Custom 0x02 = Unscaled 0x03 = Wide Screen 0x04 = Movie expand 16:9 0x05 = Auto |

Example: Current Picture Format is Widescreen on Full Display (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x3B | 0x03 | 0x1D |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x3A = Picture Format – Set |  | Command requests the display to set the specified picture format |
| DATA[1] | Picture Format | Bit 7..4 | Not used. |
|  |  | Bit 3..0 | Picture Format. 0x00 = 4:3 0x01 = Custom 0x02 = Unscaled 0x03 = Wide Screen 0x04 = Movie expand 16:9 0x05 = Auto |

The display shall respond with NAV if it receives a Picture Format that is not relevant to its Display Aspect Ratio.

The display shall ignore the [Picture Format - Set] if it receives a Picture Format that it cannot execute.

Example: Set Picture Format to Widescreen on Full Display (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x3A | 0x03 | 0x9B |

## Color Temperature

The following commands are used to get/set the color temperature.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x35 = Color Temperature – Get |  | Command requests the display to report its current color temperature. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x35 | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x35 = Color Temperature – Report |  | Command reports to the host controller the current color temperature of the display. |
| DATA[1] | Color Temperature |  | 0x00 = Custom 0x03 = Cool (10000K) 0x04 = Normal (9300K) 0x06 = Warm (6500K) |

Example: The current color temperature is set to Normal (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x35 | 0x04 | 0xnn |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x34 = Color Temperature – Set |  | Command to change the current color parameters |
| DATA[1] | Color Temperature |  | 0x00 = Custom 0x03 = Cool (10000K) 0x04 = Normal (9300K) 0x06 = Warm (6500K) |

Example: The current color temperature is set to Normal (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x34 | 0x04 | 0xnn |

## Color Temperature Parameters

The following commands are used to get/set the color parameters for specific color temperature.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x37 = Color Parameters – Get |  | Command requests the display to report its current color parameters. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x37 | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x37 = Color Parameters – Report |  | Command reports to the host controller the current color parameters of the display. |
| DATA[1] | Red color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[2] | Green color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[3] | Blue color gain value |  | 0 to 255 of the user selectable range of the display. |

Example: All color parameters are set to 128 (0x80)  (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Data[3] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x09 | 0x01 | 0x37 | 0x80 | 0x80 | 0x80 | 0xnn |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x36 = Color Parameters – Set |  | Command to change the current color parameters |
| DATA[1] | Red color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[2] | Green color gain value |  | 0 to 255 of the user selectable range of the display. |
| DATA[3] | Blue color gain value |  | 0 to 255 of the user selectable range of the display. |

Example: All color parameters are set to 128 (0x80)  (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x09 | 0x01 | 0x36 | 0x80 | 0x80 | 0x80 | 0xnn |

# MESSAGES  - AUDIO

## Volume

This command is used to set/get the Volume as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x45 = Volume – Get |  | Command requests the display to report its current Volume level |

The interface to set Software must be such that they also modify the variables representing these current parameters.

To mute the display, send Volume = 0. This command does not overwrite the system mute status of the display.

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x45 | 0xE0 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x45 = Volume – Report |  | Command reports current Volume level |
| DATA[1] | Volume. |  | 0 to 60 of the user selectable range of the display. |

Example: Current Display settings: Volume:50  (0x32) (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x45 | 0x32 | 0x52 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x44 = Volume – Set |  |  |
| DATA[1] | Volume. |  | 0 to 60 of the user selectable range of the display. |

Example: Set the Display Volume to 20 (0x14) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x44 | 0x14 | 0xF2 |

## Volume Limits

This command is used to set the volume limit (minimum, maximum and switch on volume).

### Message-Set

| Bytes | Bytes Description |  | Bits |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA[0] | 0xB8 = Volume Limits– Set |  |  |  | The 3 values must conform to the rule : Min <= Switch On <= Max |  |  |  |
| DATA[1] | Minimum Volume |  |  |  | 0 to 60 of the user selectable range of the display. |  |  |  |
| DATA[2] | Maximum Volume |  |  |  | 0 to 60 of the user selectable range of the display. |  |  |  |
| DATA[3] | Switch On Volume |  |  |  | 0 to 60 of the user selectable range of the display. |  |  |  |

Example: Set the Display to the following: 20 (0x14), 60 (0x3C), 50 (0x32) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x06 | 0x01 | 0xB8 | 0x14 | 0x3C | 0x32 | 0x02 |

## Audio Parameters

This command is used to set/get the audio parameters as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x43 = Audio Parameters – Get |  | Command requests the display to report its current audio parameters |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x43 | 0xE6 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x43 = Audio Parameters – Report |  | Command reports Audio Parameters |
| DATA[1] | Treble. |  | -8 to 8(0x2A ~ 0x3A) of the user selectable range of the display. |
| DATA[2] | Bass. |  | -8 to 8(0x2A ~ 0x3A) of the user selectable range of the display. |

Example: Current Display settings: Treble:0 (0x32) , Bass:0 (0x32)  (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0x43 | 0x32 | 0x32 | 0x67 |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x42 = Audio Parameters – Set |  | Command to change the Audio Parameters of the display |
| DATA[1] | Treble. |  | -8 to 8(0x2A ~ 0x3A) of the user selectable range of the display. |
| DATA[2] | Bass. |  | -8 to 8(0x2A ~ 0x3A) of the user selectable range of the display. |

The interface to set Software must be such that they modify the variables representing these current parameters

Example: Set the Display to the fallowing: Treble:0 (0x32) , Bass:0 (0x32)  (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x05 | 0x01 | 0x42 | 0x32 | 0x32 | 0xE1 |

# MISCELLANEOUS

## Operating Hours

The command is used to record the working hours of the display.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x0F = Misc Info - Get |  | Command requests the display to report from miscellaneous information parameters |
| DATA[1] | Subcommand |  | 0x01 = Current source status. 0x02 = Operating Hours (All other values are reserved) |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x0F | 0x02 | 0xAF |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x0F = Misc Info – Report |  | Command reports current Operating Hours |
| DATA[1] to DATA[2] | Operating Hours / Current source status. |  | Operating Hours: DATA[1] and DATA[2] form the MSByte and LSByte, respectively, of the 16-bit-wide Operational Hours value. Current source status. DATA[1]: 0x00 DATA[2]: 0x01 = signal loss / 0x02 = signal stable. |

Example: Current Display Operation Hours counter value  (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x05 | 0x01 | 0x0F | 0x00 | 0x0A | 0x21 |

## Auto Adjust

This command works for VGA (host controller) video auto adjust.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x70 = Video Alignment – Set |  | Command requests the display to make auto adjustment on VGA Input source. |
| DATA[1] | Subcommand |  | 0x40 = Auto Adjust (* All other values are reserved *) |
| DATA[2] | Reserved |  | ( reserved, fixed 0 ) |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x05 | 0x01 | 0x70 | 0x40 | 0x00 | 0x93 |

## Serial Code

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x15 = Serial Code Get |  | Command requests the display to report its Serial Code Number (Production code) 14 digits |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x15 | 0xB0 |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x15 = Serial Code – Report |  | Command reports Serial Code |
| DATA[1] | 1st Character |  | Character acc. ASCII character map (HEX) |
| DATA[2] | 2nd Character |  |  |
| DATA[3] | 3rd Character |  |  |
| DATA[14] | 14th Character |  | Character acc. ASCII character map (HEX) |

Example: Current Display settings: Serial Code = HA1A0917123456 (Display address 01)

|  | Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0x21 | 0x01 | 0x00 | 0x00 | 0x11 | 0x01 | 0x15 | 0x48 | 0x41 | 0x31 | 0x41 |

|  | Data[5] | Data[6] | Data[7] | Data[8] | Data[9] | Data[10] | Data [11] | Data[12] | Data[13] | Data[14] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0x30 | 0x39 | 0x31 | 0x37 | 0x31 | 0x32 | 0x33 | 0x34 | 0x35 | 0x36 | 0xnn |

## Temperature Sensors

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x2F = Temperature Sensor – Get |  | Command requests the display to report its value of the temperature sensors (±3°C). |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x2F | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x2F = Temperature Sensor – Report |  | Command reports Temperature sensor value |
| DATA[1] | Temperature Sensor 1 |  | 0-100 in Celsius degrees represented in hex. |

Example: Current Temp Sensor read out: Sensor 1 = 28°C (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x2F | 0x1C | 0xnn |

## Tiling

The command is used to set/get the tiling status as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x23 = Tiling – Get |  | Command requests the display to report Tiling status. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x23 | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x23 = Tiling – Report |  | Command reports Tiling Setting |
| DATA[1] | Enable |  | 0x00 = No 0x01 = Yes |
| DATA[2] | Frame comp. |  | 0x00 = No 0x01 = Yes |
| DATA[3] | Position |  | 0x01 = position 1 0x02 = position 2 ... See Note 1 |
| DATA[4] | V Monitors, H Monitors |  | 0x00 = don’t care 0x01 = V Monitors =1, H Monitors =1 0x02 = V Monitors =1, H Monitors =2 … See Note 2 |

Note 1:

(1) The maximum Position value is 100 (hexadecimal value is 0x64).

(2) The Position is counted from left to right, then up to down in the Tiling Wall.

Example: See Figure 1 for the hexadecimal Position value in a 4x3 (H Monitors x V Monitors) Tiling Wall.

Example: See Figure 2 for the hexadecimal Position value in a 5x5 (H Monitors x V Monitors) Tiling Wall.

Note 2:

(1) The maximum H Monitors are 10 and the maximum V Monitors are 10. The formulas for DATA[4], V Monitors, and H Monitors are as follows:

H Monitors = MOD(Data[4], 10)          (Data[4] ÷ 10, take the remainder)

V Monitors = INT(Data[4], 10) + 1        (Data[4] ÷ 10, take the quotient and plus one)

Data[4]= (V Monitors – 1) x 10 + H Monitors

Example: If H Monitors = 10 and V Monitors = 6, the Data[4] value will be (6–1) x 10 + 10 = 60

Example 1:

Set the display as follows:

Tiling enabled: Yes

Frame comp.: No

Position: 2

H Monitors: 4

V Monitors: 3

Data[4] value will be: (3–1) x 10 + 4 = 24 (hex value: 0x18)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x07 | 0x01 | 0x23 | 0x01 | 0x00 | 0x02 | 0x18 | 0xnn |

Figure1. The hexadecimal Position value in a 4x3 (H Monitors x V Monitors) Tiling Wall.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x22 = Tiling – Set |  | Command reports Tiling Setting |
| DATA[1] | Enable |  | 0x00 = No 0x01 = Yes |
| DATA[2] | Frame comp. |  | 0x00 = No 0x01 = Yes 0x02 = don’t overwrite (keep previous value) |
| DATA[3] | Position |  | 0x00 = don’t overwrite (keep previous value) 0x01 = position 1 0x02 = position 2 … See Note 1 at 8.6.2 |
| DATA[4] | V Monitors, H Monitors |  | 0x00 = don’t overwrite (keep previous value) 0x01 = V Monitors =1, H Monitors =1 0x02 = V Monitors =1, H Monitors =2 … See Note 2 at 8.6.2 |

Example 1:

Set the display as follows:

Tiling enabled: Yes

Frame comp.: No

Position: 2

H Monitors: 5

V Monitors: 5

Data[4] value will be (5–1) x 10 + 5 = 45 (hex value: 0x2D)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x07 | 0x01 | 0x22 | 0x01 | 0x00 | 0x02 | 0x2D | 0xnn |

Figure2. The hexadecimal Position value in a 5x5 (H Monitors x V Monitors) Tiling Wall.

Example 2:

Set the display as follows:

Tiling enabled: Yes

Frame comp., Position, H Monitors, V Monitors: Keep as before

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Data[2] | Data[3] | Data[4] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x07 | 0x01 | 0x22 | 0x01 | 0x02 | 0x00 | 0x00 | 0xnn |

## Backlight Level

This command is used to set/get the Backlight level as it is defined as below.

### Message-Get

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x31 = Backlight level – Get |  | Command requests the display to report its current Backlight level. |

Example: (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0x31 | 0xnn |

### Message-Report

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x31 = Backlight level – Report |  | Command reports current Backlight level |
| DATA[1] | Backlight value |  | 0 to 100 of the user selectable range of the display. |

Example: Backlight:90 (0x5A)   (Display address 01)

| Header | Monitor ID | Category | Page | Length | Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x21 | 0x01 | 0x00 | 0x00 | 0x04 | 0x01 | 0x31 | 0x5A | 0xnn |

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0x30 = Backlight level – Set |  | Command to change the Backlight level of the display. |
| DATA[1] | Backlight value |  | 0 to 100 of the user selectable range of the display. |

Example: Set Backlight:90 (0x5A) (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0x42 | 0x5A | 0xnn |

## Factory Reset

The command is used to reset all you customized settings to the factory defaults.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xFE = Factory Reset |  | Command to do the Factory Reset of the display |

Example: Set Factory Settings (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x03 | 0x01 | 0xFE | 0x5B |

## IR Remote Command

The command is used to send IR Key to Control display.

### Message-Set

| Bytes | Bytes Description | Bits | Description |
| --- | --- | --- | --- |
| DATA[0] | 0xFD = IR Remote Command |  | Command to simulate the IR Remote to send IR Key to display |
| DATA[1] | IR KEY |  | 0xA0: Power 0xA1: Menu 0xA2: Input 0xA3: Vol_Up 0xA4: Vol_Down 0xA5: Mute 0xA6: Cursor_Up 0xA7: Cursor_Down 0xA8: Cursor_Left 0xA9: Cursor_Right 0xB1: OK 0xB2: Return 0xC1: Red 0xC2: Green 0xC3: Yellow 0xC4: Blue 0xD1: Format 0xD2: Info 0x00: Btn_0 0x01: Btn_1 0x02: Btn_2 0x03: Btn_3 0x04: Btn_4 0x05: Btn_5 0x06: Btn_6 0x07: Btn_7 0x08: Btn_8 0x09: Btn_9 |

Example: Send Power Key (Display address 01)

| Header | Monitor ID | Category | Code0 | Code1 | Length | Data Control | Data[0] | Data[1] | Checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xA6 | 0x01 | 0x00 | 0x00 | 0x00 | 0x04 | 0x01 | 0xFD | 0xA0 | 0xFF |

# Command summary

| Command name | Set Command | Get Command | Command Code | Remarks |
| --- | --- | --- | --- | --- |
| Communication Control |  |  | 0x00 | Generic report |
| Platform and version labels |  |  | 0xA2 |  |
| Power state get |  |  | 0x19 |  |
| Power state set |  |  | 0x18 |  |
| User Input Control get |  |  | 0x1D |  |
| User Input Control set |  |  | 0x1C |  |
| Power at cold start set |  |  | 0xA3 |  |
| Power at cold start get |  |  | 0xA4 |  |
| Input Source |  |  | 0xAC |  |
| Current Source |  |  | 0xAD |  |
| Auto Signal Detecting Get |  |  | 0xAF |  |
| Auto Signal Detecting Set |  |  | 0xAE |  |
| Video parameters get |  |  | 0x33 | Brightness, etc. |
| Video parameters set |  |  | 0x32 |  |
| Color Temperature Get |  |  | 0x35 |  |
| Color Temperature Set |  |  | 0x34 |  |
| Color Parameters Get |  |  | 0x37 |  |
| Color Parameters Set |  |  | 0x36 |  |
| Picture Format get |  |  | 0x3B |  |
| Picture Format set |  |  | 0x3A |  |
| Volume get |  |  | 0x45 |  |
| Volume set |  |  | 0x44 |  |
| Volume limits |  |  | 0xB8 |  |
| Audio parameters get |  |  | 0x43 |  |
| Audio parameters set |  |  | 0x42 |  |
| Miscellaneous info |  |  | 0x0F | Signal status |
|  |  |  |  | Operating hours |
| Auto Adjust |  |  | 0x70 | VGA only |
| Serial Code Get |  |  | 0x15 |  |
| Temperature Get |  |  | 0x2F |  |
| Tiling Get |  |  | 0x23 |  |
| Tiling Set |  |  | 0x22 |  |
| Backlight Level Get |  |  | 0x31 |  |
| Backlight Level Set |  |  | 0x30 |  |
| Factory Reset |  |  | 0xFE |  |
| IR Remote Control |  |  | 0xFD |  |